Networking Devices

- Hub

- Bridge

- Switch

- Router

- Gateway

- Hub :-

- LAN device

- Works on Layer- 1(Physical Layer)

- Always broadcast

- Electrical or binary signals can be sent by hub

- Doesn’t store MAC addresses

- Half duplex (only one can send packet at once)

- Single collision domain

- Types :

  - Active hub :- acts as a repeater(amplify analog signals or regenerate binary signals), and It needs electricity

  - Passive hub :- Only propagates the signal

Sender

Hub

Receiver

- Bridge :-

- LAN device

- Layer-2 device (Data link layer)

- Stores MAC addresses in a table.

- Used to connect multiple LAN segments or LAN segments.

- It divides the LAN into 2 parts.

- Bridge inspects the incoming traffic and decides whether to forward or reject it. It checks source and destination mac address.

- Working :-

- Bridge firstly broadcast to the both LAN segments and once the MAC of sender and receiver is known then it only broadcasts to the segment where receiver is present.

LAN -2

LAN -1

- Scenario -1 : A wants to send message to H.

- When A sends message, the bridge broadcasts msg to the both sides, and once H sends acknowledgement, it stores the MAC addresses of both A & H along with port numbers.

| Port No. | MAC address |
| --- | --- |
| 1 | A |
| 2 | H |

- Scenario – 2 : B wants to talk to G.

- When B sends message, bridge broadcasts msg to both the segments, once G acknowledges the msg, it stores the MAC of both to the table.

| Port No. | MAC address |
| --- | --- |
| 1 | A,B |
| 2 | H,G |

- Scenario – 3 : A wants to talk to E.

- When A sends msg to E, the bridge still broadcasts even though belonging to the port-1 as the MAC of E is unknown. Once E acknowledges the msg, its MAC address is stored in table.

| Port No. | MAC address |
| --- | --- |
| 1 | A,B,E |
| 2 | H,G |

- Scenario – 4 : A wants to talk E again.

- As mac of both are known, bridge will broadcast the message to the LAN segment 1 only.

- Switch (Multiport Bridge) :-

- Layer-2 device (Data-Link layer)

- Full duplex

- Maintains CAM(content accessible memory) table.

- First broadcast, then unicast and multicast.

- Every port of switch has separate collision domain.

- Switch has only one broadcast domain.

- 6/8/24/48 ports available.

- Uses ASICS (Application Specific ICs)

- Slower

- Modes of Switches :

- Store & forward switch :

- The switch buffers and verifies each frame, before forwarding it.

- It’s a little bit slow.

- Cut through switch :-

- It only checks MAC address and then forwards the frame.

- There’s no error checking.

- Fragment free switch :-

- A methods that retains merits of both store& forward switch & cut through switch.

- It only checks first 64 bytes of frame.

- Adaptive switching :

- A method of automatically selecting between other modes of switching.

- Router :-

- Layer – 3 device

- WAN device

- Internetworking device

- Connects two or more networks.

- In router, Each port has its own broadcasting domain.

- Uses IP addresses to maintain Routing table.

- 2/4/8 ports.

- Faster compare to switches.

- Gateway :-

- Gateway is an entry/exit point of network. Sometimes it is implemented at switch.