- Why does the BJT enters into Saturation Region even though VCE  ⊕ve being?

- The bias of CB junction depends upon the difference of two voltages not merely the polarity of VCE.

- Even with ⊕ve VCE, the transistor can enter the saturation region because the collector voltage(VC) can drop below the base voltage(VB).

- NL & PH – Forward bias

- NH & PL – Reverse bias

- To forward bias the CB junction, N-type Collector needs to be at Low voltage and P-type Base needs to be at High Voltage.

VCB = VCE – VBE

- Active Region : VCE > VBE (e.g., VCE = 5v and VBE = 0.7v, then VCB = 4.3v) , The collector is more ⊕ve than the base, so the CB is reverse biased.

- Saturation Region : VCE < VBE (e.g., VCE = 0.3v and VBE = 0.7v, then VCB = - 0.4v), The Collector is at lesser voltage and hence Forward biased.

- Even though VCE still being ⊕ve related to the ground(Emitter), the collector is at lower potential than the base and hence Forward biased.

- Why does VCE drop so LOW ?

- In real life, VCE isn’t just a battery, it’s the leftover voltage after IC passes through the Load Voltage RC.

VCE = VCC - IC *RC

- As you increase the input current IB, IC increases. This causes a larger voltage drop across RC. Eventually, IC *RC  becomes so large that VCE is pushed down toward 0.2V or 0.3V. At that point, the Collector can no longer "sink" more current effectively because it is no longer pulling electrons away from the base—it's actually pushing them back!

- Note : VCE is usually quoted as 0.2v. At this stage, the CB junction is forward biased by about 0.5v (0.7v - 0.2v = 0.5v), which is enough to flood the base with carriers and stop the normal "active" flow.

| Region | VBE Status |  | VCE Vs. VBE | CB Junction Bias |
| --- | --- | --- | --- | --- |
| Cut-off |  | Reverse (< 0.7v) | N/A | Reverse |
| Active |  | Forward (≈ 0.7v) | VCE > VBE | Reverse |
| Saturation |  | Forward (≈ 0.7V) | VCE < VBE | Forward |