## Why do we join a Resistor parallelly across the practical Current source ? What is the significance of that ? why not in series instead of parallel ?

- We represent a Practical Current Source as an ideal current source with a resistor (Rs) connected in parallel.

- The Leakage Path: This parallel resistor represents the internal losses of the source.

- Current Division: When you connect an external load, the total current from the "ideal" part of the source has to split. Some goes to your device (the load), and some "leaks" through the internal parallel resistor.

- Real-world behaviour : As you increase the resistance of your external load, it becomes harder for current to flow through it. Consequently, more current is forced through that internal parallel resistor, and the current delivered to your load drops. This perfectly mimics how real power supplies behave.

## 1. Why not in Series?

This is the most common point of confusion. Let's look at what happens if we put the resistor in series:

- Current stays the same: In a series circuit, the current is identical at every point. If an ideal current source is pushing 5A, and you put a resistor in series with it, it is still pushing 5A through that resistor and into your load.

- No "Loss" signal: A series resistor would change the voltage required to push the current, but it wouldn't explain why the current drops when the load changes.

- The Voltage Source Comparison: We use a series resistor to represent a Practical Voltage Source (like an AA battery). In a battery, the voltage drops as you draw more current because of internal resistance in the path.

Key Takeaway: We use Parallel for Current Sources because we want to model how current "splits" (leaks). We use Series for Voltage Sources because we want to model how voltage "drops."

## 2. The Significance: Maximum Power Transfer

Understanding this parallel resistance is crucial for engineers.

- Efficiency: For a current source to be "good," that internal parallel resistance needs to be as high as possible. The higher it is, the less current leaks through it, and the more reaches your load.

- The Math: If Is is the source current, RS is the internal resistance, and RL is the load, the current reaching the load (IL) is calculated using the current divider rule:
IL= Is×RsRs+ RL

### Comparison Table

| Feature | Practical Voltage Source | Practical Current Source |
| --- | --- | --- |
| Internal Resistance Connection | Series | Parallel |
| Ideal Internal Resistance | 0 Ω (Zero) | ∞ Ω (Infinite) |
| What it models | Voltage drop under load | Current leakage under load |